Auf allen Maschinen:

sudo apt update

sudo apt install bridge-utils vlan

Konfiguration der Clients

nano /etc/network/interfaces

NWTK2

auto enp0s8.10

iface enp0s8.10 inet static

address 192.168.10.1

netmask 255.255.255.0

gateway 192.168.10.254

vlan-raw-device enp0s8

NWTK3

auto enp0s8.20

iface enp0s8.20 inet static

address 192.168.20.1

netmask 255.255.255.0

gateway 192.168.20.254

vlan-raw-device enp0s8

Server

VLAN 10

auto enp0s8.10

iface enp0s8.10 inet static

address 192.168.10.254

netmask 255.255.255.0

vlan-raw-device enp0s8

VLAN 20

auto enp0s8.20

iface enp0s8.20 inet static

address 192.168.20.254

netmask 255.255.255.0

vlan-raw-device enp0s8

Routing temporär aktivieren

echo 1 > /proc/sys/net/ipv4/ip_forward

cat /proc/sys/net/ipv4/ip_forward

Routing permanent aktivieren

nano /etc/sysctl.conf

net.ipv4.ip_forward=1

Routing Tabelle anzeigen

ip route show

Ping

Vergleich

VLAN

(Linux)

auto enp0s8.10

iface enp0s8.10 inet static

address 192.168.10.254

netmask 255.255.255.0

vlan-raw-device enp0s8

(Cisco)

interface g0/0.10

encapsulation dot1Q 10

ip address 192.168.10.254 255.255.255.0

Routing aktivieren

(Linux)

echo 1 > /proc/sys/net/ipv4/ip_forward

(Cisco)

ip routing

Routing Tabelle

(Linux)

ip route show

(Cisco)

show ip route

Ip mit ip-Befehl konfigurieren

Ip setzten

sudo ip addr add 192.168.10.1/24 dev enp0s8.10

Interface aktivieren

sudo ip link set enp0s8.10 up

Default Gateway setzen

sudo ip route add default via 192.168.10.254

VLAN erstellen

sudo ip link add link enp0s8 name enp0s8.10 type vlan id 10

Routing Tabelle

(Linux)

192.168.10.0/24 dev enp0s8.10 proto kernel scope link src 192.168.10.254

192.168.20.0/24 dev enp0s8.20 proto kernel scope link src 192.168.20.254

Erreichbar über

Schickt zu Router mit ip

(Cisco)

C 192.168.10.0/24 is directly connected, GigabitEthernet0/0.10

C 192.168.20.0/24 is directly connected, GigabitEthernet0/0.20

Über das Subinterface