Clients konfigurieren

Client 1a (VLAN 10)

nano /etc/network/interfaces

auto enp0s8

iface enp0s8 inet manual

up ip link add link enp0s8 name enp0s8.10 type vlan id 10

up ip link set enp0s8 up

up ip link set enp0s8.10 up

up ip addr add 192.168.10.2/24 dev enp0s8.10

up ip route add default via 192.168.10.1

Client 1b (VLAN 20)

nano /etc/network/interfaces

auto enp0s8

iface enp0s8 inet manual

up ip link add link enp0s8 name enp0s8.20 type vlan id 20

up ip link set enp0s8 up

up ip link set enp0s8.20 up

up ip addr add 192.168.20.2/24 dev enp0s8.20

up ip route add default via 192.168.20.1

Client 2a (VLAN 30)

nano /etc/network/interfaces

auto enp0s8

iface enp0s8 inet manual

up ip link add link enp0s8 name enp0s8.30 type vlan id 30

up ip link set enp0s8 up

up ip link set enp0s8.30 up

up ip addr add 192.168.30.2/24 dev enp0s8.30

up ip route add default via 192.168.30.1

Client 2b (VLAN 40)

nano /etc/network/interfaces

auto enp0s8

iface enp0s8 inet manual

up ip link add link enp0s8 name enp0s8.40 type vlan id 40

up ip link set enp0s8 up

up ip link set enp0s8.40 up

up ip addr add 192.168.40.2/24 dev enp0s8.40

up ip route add default via 192.168.40.1

Router 1 konfigurieren

VLAN Interfaces usw.

nano /etc/network/interfaces

auto enp0s8

iface enp0s8 inet manual

up ip link add link enp0s8 name enp0s8.10 type vlan id 10

up ip link add link enp0s8 name enp0s8.20 type vlan id 20

up ip link set enp0s8 up

up ip link set enp0s8.10 up

up ip link set enp0s8.20 up

up ip addr add 192.168.10.1/24 dev enp0s8.10

up ip addr add 192.168.20.1/24 dev enp0s8.20

up ip addr add 10.0.0.1/30 dev enp0s8

IP-Forwarding

sudo nano /etc/sysctl.conf       ==       net.ipv4.ip_forward=1

Router 2 konfigurieren

nano /etc/network/interfaces

VLAN Interfaces usw.

auto lo

iface lo inet loopback

auto enp0s8

iface enp0s8 inet manual

up ip link add link enp0s8 name enp0s8.30 type vlan id 30

up ip link add link enp0s8 name enp0s8.40 type vlan id 40

up ip link set enp0s8 up

up ip link set enp0s8.30 up

up ip link set enp0s8.40 up

up ip addr add 192.168.30.1/24 dev enp0s8.30

up ip addr add 192.168.40.1/24 dev enp0s8.40

up ip addr add 10.0.0.2/30 dev enp0s8

IP-Forwarding

sudo nano /etc/sysctl.conf       ==       net.ipv4.ip_forward=1

RIP auf beiden Routern

Installation

sudo apt install frr -y

Daemons aktivieren

sudo nano /etc/frr/daemons

zebra=yes  (einfach dazu schreiben)

ripd=yes    (das gibt es schon einfach zu yes machen)

Dienst starten

sudo systemctl restart frr

Router 1 RIP

sudo vtysh

----------------

configure terminal

router rip

version 2

network 192.168.10.0/24

network 192.168.20.0/24

network 10.0.0.0/30

no auto-summary

end

write

copy running-config startup-config

Router 2 RIP

sudo vtysh

--------------

configure terminal

router rip

version 2

network 192.168.30.0/24

network 192.168.40.0/24

network 10.0.0.0/30

no auto-summary

end

write

copy running-config startup-config

Tests

Ping

Client 1a:

ping 192.168.40.2

Client 2a:

ping 192.168.10.2

Traceroute am Client 1a

traceroute 192.168.40.2

Routing Tabellen

Linux:

ip route show

RIP:

Im vtysh

show ip route